Supplementary materials

Title: Should we delay leaf water potential measurements after excision? Dehydration or equilibration?

Authors: Alicia V Perera-Castro, Jaime Puértolas, Beatriz Fernández-Marín, Águeda M González-Rodríguez

Figure S1. Variation of relative water content (RWC) of leaves as a consequence of water potential measurement in three studied species. Leaves were weighted immediately after excision and immediately after depressurization (2-3 minutes of difference). Water potential (-0.55 ± 0.02, -0.72 ± 0.02 and -0.46 ± 0.01 MPa, for Solanum nigrum, Prunus lusitanica and Morella faya, respectively) was measured by the same operator with a pressure chamber (model 1505D-EXP, PMS Instrument Company, Albany, USA) applying a stable rate of pressurization (0.01 MPa s-1). A moist paper was placed inside pressure chamber to minimize dehydration during measurements. The averaged drop of RWC (ΔRWC = RWC after excision – RWC after water potential determination) is shown in the upper side of each panel.

Table S1. Variation of water potential (Ψw, MPa) and relative water content (RWC, %) after one hour of storage under either the “simple humid bag” method (Bag) or the “vaseline plus humid bag” (Bag+Vaseline) method of leaf storage. Note that ΔΨw was calculated comparing values from two different adjacent leaves (Ψw,0 and Ψw,1h) collected simultaneously, while ΔRWC compares values taken in the same leaf (RWC0 and RWC1h). ID corresponds to the branch were leaves of both treatments were collected.

| Parameter | Storage | ID | Solanum nigrum |  |  | Morella faya |  |  | Prunus lusitanica |  |  | Scrophularia glabrata |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1h | Δ | 0 | 1h | Δ | 0 | 1h | Δ | 0 | 1h | Δ |
| Ψw | Bag | 1 | -0.59 | -0.87 | -0.28 | -0.91 | -1.19 | -0.28 | -0.42 | -1.4 | -0.98 | -0.7 | -1.26 | -0.56 |
|  |  | 2 | -0.65 | -1.02 | -0.37 | -0.89 | -1.08 | -0.19 | -0.42 | -0.89 | -0.47 | -0.85 | -0.98 | -0.13 |
|  |  | 3 | -0.61 | -1.03 | -0.42 | -0.96 | -1.15 | -0.19 | -0.65 | -1.14 | -0.49 | -0.76 | -0.95 | -0.19 |
|  |  | 4 | -0.56 | -0.97 | -0.41 | -1.06 | -1.18 | -0.12 | -0.43 | -0.95 | -0.52 | -0.78 | -0.96 | -0.18 |
|  |  | 5 | -0.64 | -0.92 | -0.28 | -0.82 | -1.08 | -0.26 | -0.41 | -0.89 | -0.48 | -0.85 | -1.09 | -0.24 |
|  |  | MEAN | -0.61 | -0.96 | -0.35 | -0.92 | -1.13 | -0.21 | -0.46 | -1.05 | -0.59 | -0.78 | -1.04 | -0.26 |
|  |  | SE | 0.01 | 0.03 | 0.03 | 0.03 | 0.02 | 0.02 | 0.04 | 0.09 | 0.09 | 0.05 | 0.05 | 0.07 |
|  | Bag+Vaseline | 1 | -0.47 | -0.58 | -0.11 | -0.93 | -0.89 | 0.04 | -0.41 | -0.78 | -0.37 | -0.79 | -0.89 | -0.1 |
|  |  | 2 | -0.60 | -0.76 | -0.16 | -0.84 | -0.82 | 0.02 | -0.53 | -0.85 | -0.32 | -0.77 | -0.84 | -0.07 |
|  |  | 3 | -0.59 | -0.74 | -0.15 | -0.80 | -0.65 | 0.15 | -0.39 | -0.72 | -0.33 | -0.75 | -0.89 | -0.14 |
|  |  | 4 | -0.61 | -0.72 | -0.11 | -0.82 | -0.82 | 0.00 | -0.41 | -0.75 | -0.34 | -0.8 | -0.93 | -0.13 |
|  |  | 5 | -0.59 | -0.77 | -0.18 | -0.71 | -0.77 | -0.06 | -0.38 | -0.89 | -0.51 | -0.7 | -0.88 | -0.18 |
|  |  | MEAN | -0.57 | -0.71 | -0.14 | -0.82 | -0.79 | 0.03 | -0.42 | -0.79 | -0.37 | -0.76 | -0.88 | -0.12 |
|  |  | SE | 0.02 | 0.03 | 0.01 | 0.03 | 0.03 | 0.03 | 0.02 | 0.03 | 0.03 | 0.01 | 0.01 | 0.01 |
| RWC | Bag | 1 | 95.8 | 89.2 | 6.6 | 89.0 | 87.4 | 1.6 | 92.4 | 89.8 | 2.6 | 89.7 | 88.4 | 1.3 |
|  |  | 2 | 89.6 | 80.9 | 8.7 | 90.0 | 89.2 | 0.7 | 90.7 | 90.1 | 0.6 | 103.1 | 101.8 | 1.3 |
|  |  | 3 | 98.4 | 87.6 | 10.8 | 89.7 | 88.5 | 1.2 | 91.8 | 90.4 | 1.4 | 90.2 | 88.5 | 1.7 |
|  |  | 4 | 103.2 | 94.7 | 8.5 | 86.8 | 85.2 | 1.5 | 107.5 | 106.3 | 1.2 | 96.4 | 94.4 | 2.0 |
|  |  | 5 | 96.4 | 88.5 | 7.9 | 93.4 | 92.0 | 1.4 | 84.9 | 84.1 | 0.8 | 98.4 | 96.3 | 2.1 |
|  |  | MEAN | 96.6 | 88.1 | 8.5 | 89.7 | 88.4 | 1.4 | 93.4 | 92.1 | 1.3 | 95.5 | 93.8 | 1.7 |
|  |  | SE | 2.1 | 2.2 | 0.6 | 1.0 | 1.11 | 0.1 | 3.7 | 3.7 | 0.3 | 2.5 | 2.5 | 0.2 |
|  | Bag+Vaseline | 1 | 95.0 | 94.2 | 0.8 | 88.1 | 87.3 | 0.8 | 95.5 | 94.9 | 0.6 | 88.9 | 88.2 | 0.7 |
|  |  | 2 | 90.6 | 89 | 1.6 | 92.4 | 91.7 | 0.7 | 93.7 | 93.1 | 0.6 | 100.2 | 99 | 1.2 |
|  |  | 3 | 99.0 | 98.3 | 0.7 | 90.2 | 89.7 | 0.5 | 91.5 | 90.3 | 1.2 | 91.6 | 90.4 | 1.2 |
|  |  | 4 | 97.0 | 96.0 | 1.0 | 87.8 | 87.4 | 0.4 | 106.2 | 105.5 | 0.7 | 96.5 | 96.2 | 0.3 |
|  |  | 5 | 97.4 | 96.7 | 0.7 | 93.6 | 93.6 | 0.0 | 100.9 | 100.2 | 0.7 | 97.8 | 96.8 | 1.0 |
|  |  | MEAN | 95.8 | 94.8 | 1.0 | 90.4 | 89.9 | 0.5 | 97.5 | 96.8 | 0.7 | 95.0 | 94.1 | 0.9 |
|  |  | SE | 1.4 | 1.6 | 0.1 | 1.1 | 1.2 | 0.1 | 2.6 | 2.7 | 0.1 | 2.0 | 2.0 | 0.1 |

Table S2. Statistics results about the difference between ΔΨw and zero (one sample t-test, experiment 1).

| Species | Storage | ΔΨw | One-sample t-test |  |
| --- | --- | --- | --- | --- |
|  |  |  | t | P |
| Solanum nigrum | Bag | -0.35 ± 0.03 | -11.51 | <0.001 |
|  | Bag+Vaseline | -0.14 ± 0.01 | -10.19 | <0.001 |
| Morella faya | Bag | -0.21 ± 0.02 | -7.29 | 0.001 |
|  | Bag+Vaseline | 0.03 ± 0.03 | 0.87 | 0.431 |
| Prunus lusitanica | Bag | -0.59 ± 0.09 | -5.97 | 0.003 |
|  | Bag+Vaseline | -0.37 ± 0.03 | -10.68 | <0.001 |
| Scrophularia glabrata | Bag | -0.26 ± 0.07 | -3.37 | 0.027 |
|  | Bag+Vaseline | -0.12 ± 0.01 | -6.66 | 0.002 |

Table S3. Statistic results about the effect of the storage on the changes of ΔΨw or ΔRWC with time (two-way repeated measures ANOVA, experiment 1).

| Species | Parameter | Storage |  | Time |  | Storage x Time |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | F | P | F | P | F | P |
| Solanum nigrum | ΔΨw | 52.23 | 0.002 | 247.50 | <0.001 | 34.72 | 0.004 |
|  | ΔRWC | 3.93 | 0.118 | 181.67 | <0.001 | 114.94 | <0.001 |
| Morella faya | ΔΨw | 35.56 | 0.004 | 13.63 | 0.021 | 34.04 | 0.004 |
|  | ΔRWC | 4.75 | 0.095 | 98.18 | <0.001 | 13.94 | 0.020 |
| Prunus lusitanica | ΔΨw | 3.78 | 0.124 | 87.52 | <0.001 | 4.07 | 0.114 |
|  | ΔRWC | 2.03 | 0.227 | 32.43 | 0.005 | 2.29 | 0.204 |
| Scrophularia glabrata | ΔΨw | 8.34 | 0.045 | 24.63 | 0.008 | 2.81 | 0.169 |
|  | ΔRWC | 0.04 | 0.846 | 165.49 | <0.001 | 8.41 | 0.044 |

Table S4. Evolution of water potential (Ψw) and relative water content (RWC) at 25, 50 or 240 min after excision of leaves stored with the “vaseline plus humid bag” method. Note that ΔΨw was calculated comparing values from two different adjacent leaves collected simultaneously (Ψw,0 and Ψw,x, being x=20, 50 or 250 min), while ΔRWC compares values taken in the same leaf (RWC0 and RWCx). ID corresponds to the branch were leaves of both treatments were collected.

| Parameter | ID | Viburnum rigidum |  |  |  | Solanum nigrum |  |  |  | Prunus lusitanica |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 20 | 50 | 250 | 0 | 20 | 50 | 250 | 0 | 20 | 50 | 250 |
| Ψw | 1 | -0.39 | -0.40 | -0.40 | -0.46 | -0.65 | -0.64 | -0.68 | -0.79 | -0.49 | -0.51 | -0.51 | -0.79 |
|  | 2 | -0.48 | -0.55 | -0.55 | -0.6 | -0.67 | -0.56 | -0.78 | -0.77 | -0.55 | -0.60 | -0.58 | -0.67 |
|  | 3 | -0.39 | -0.47 | -0.45 | -0.64 | -0.57 | -0.62 | -0.59 | -0.76 | -0.48 | -0.50 | -0.58 | -0.62 |
|  | 4 | -0.46 | -0.45 | -0.47 | -0.54 | -0.82 | -0.71 | -0.91 | -0.93 | -0.49 | -0.55 | -0.52 | -0.66 |
|  | 5 | -0.51 | -0.51 | -0.48 | -0.52 | -0.61 | -0.63 | -0.70 | -0.82 | -0.47 | -0.45 | -0.42 | -0.58 |
|  | MEAN | -0.44 | -0.47 | -0.47 | -0.55 | -0.66 | -0.63 | -0.73 | -0.81 | -0.49 | -0.52 | -0.52 | -0.66 |
|  | SE | 0.02 | 0.02 | 0.02 | 0.03 | 0.04 | 0.02 | 0.04 | 0.03 | 0.01 | 0.02 | 0.02 | 0.03 |
| ΔΨw | 1 | - | -0.01 | -0.01 | -0.07 | - | 0.01 | -0.03 | -0.14 | - | -0.02 | -0.02 | -0.30 |
|  | 2 | - | -0.07 | -0.07 | -0.12 | - | 0.11 | -0.11 | -0.10 | - | -0.05 | -0.03 | -0.12 |
|  | 3 | - | -0.08 | -0.06 | -0.25 | - | -0.05 | -0.02 | -0.19 | - | -0.02 | -0.1 | -0.14 |
|  | 4 | - | 0.01 | -0.01 | -0.08 | - | -0.11 | -0.09 | -0.11 | - | -0.06 | -0.03 | -0.17 |
|  | 5 | - | 0.0 | 0.03 | -0.01 | - | -0.02 | -0.09 | -0.21 | - | 0.02 | 0.05 | -0.11 |
|  | MEAN | - | -0.03 | -0.02 | -0.10 | - | 0.03 | -0.06 | -0.15 | - | -0.02 | -0.02 | -0.16 |
|  | SE | - | 0.01 | 0.01 | 0.04 | - | 0.03 | 0.02 | 0.02 | - | 0.01 | 0.02 | 0.03 |
| RWC0 | 1 | 98.1 | 98.7 | 96.2 | 97.0 | 91.5 | 94.2 | 94.9 | 96.2 | 96.5 | 97.4 | 95.0 | 95.2 |
|  | 2 | 97.8 | 96.0 | 95.9 | 96.6 | 94.2 | 95.7 | 93.6 | 96.1 | 97.6 | 97.1 | 97.6 | 98.1 |
|  | 3 | 98.6 | 96.9 | 98.3 | 97.9 | 90.1 | 94.5 | 95.7 | 96.4 | 97.8 | 98.6 | 98.1 | 98.8 |
|  | 4 | 97.5 | 96.3 | 96.1 | 98.4 | 91.7 | 95.2 | 92.8 | 92.5 | 97.4 | 97.7 | 98.1 | 96.9 |
|  | 5 | 98.1 | 96.6 | 97.2 | 96.9 | 93.1 | 95.6 | 95.1 | 95.1 | 97.5 | 97.2 | 97.6 | 91.0 |
|  | MEAN | 98.0 | 96.9 | 96.7 | 97.3 | 92.1 | 95.0 | 94.4 | 95.2 | 97.3 | 97.6 | 97.2 | 96.0 |
|  | SE | 0.1 | 0.4 | 0.4 | 0.3 | 0.7 | 0.3 | 0.5 | 0.7 | 0.2 | 0.2 | 0.6 | 1.3 |
| RWCx | 1 | - | 98.7 | 96.4 | 96.6 | - | 94.2 | 94.6 | 94.4 | - | 97.4 | 95.2 | 94.8 |
|  | 2 | - | 95.8 | 96.0 | 95.6 | - | 95.4 | 93.6 | 94.5 | - | 97.2 | 97.7 | 97.6 |
|  | 3 | - | 96.7 | 98.1 | 97.5 | - | 94.0 | 93.9 | 94.5 | - | 98.1 | 97.8 | 98.1 |
|  | 4 | - | 96.3 | 96.0 | 97.9 | - | 95.4 | 92.2 | 90.7 | - | 97.4 | 98.2 | 96.7 |
|  | 5 | - | 96.8 | 97.2 | 97.2 | - | 95.9 | 94.9 | 93.5 | - | 97.2 | 97.5 | 90.5 |
|  | MEAN | - | 96.8 | 96.7 | 96.9 | - | 94.9 | 93.8 | 93.5 | - | 97.4 | 97.3 | 95.5 |
|  | SE | - | 0.5 | 0.4 | 0.4 | - | 0.3 | 0.4 | 0.7 | - | 0.1 | 0.5 | 1.3 |
| ΔRWC | 1 | - | 0.0 | 0.2 | -0.4 | - | 0.0 | -0.3 | -1.8 | - | 0.0 | 0.2 | -0.4 |
|  | 2 | - | -0.2 | 0.1 | -1 | - | -0.3 | 0.0 | -1.6 | - | 0.1 | 0.1 | -0.5 |
|  | 3 | - | -0.2 | -0.2 | -0.4 | - | -0.5 | -1.8 | -1.9 | - | -0.5 | -0.3 | -0.7 |
|  | 4 | - | 0.0 | -0.1 | -0.5 | - | 0.2 | -0.6 | -1.8 | - | -0.3 | 0.1 | -0.2 |
|  | 5 | - | 0.2 | 0.0 | 0.3 | - | 0.3 | -0.2 | -1.6 | - | 0.0 | -0.1 | -0.5 |
|  | MEAN | - | -0.0 | 0.0 | -0.4 | - | -0.0 | -0.5 | -1.7 | - | -0.1 | 0.0 | -0.4 |
|  | SE | - | 0.0 | 0.0 | 0.2 | - | 0.1 | 0.3 | 0.0 | - | 0.1 | 0.1 | 0.1 |